EURO PROFILE CYLINDER

Definitions of cylinder lengths:

The length will be declared up to the middle of the cylinder screw.  The outside “A” and the inside “B” for profile cylinders has to be declared separately.

Function:

1.	Double Function

2.	Single Entry Function

3.	Radial Function

4.	Privacy Function

Key For Keying Systems

1.	Grand Master Key

2.	Master Key

3.	Group Key

4.	Key Aliked

5.	Single Key

Key Alike System

Two or more cylinders have the same individual key.  They may or may not be part of keying system (the same key can operate two or more locks.)

Notice:

Key Alike (KA) can operate locks no.1-4

EURO PROFILE CYLINDER

Master Key System

All cylinders have their own keys.  The single-key passes only one cylinder.  The master-key passes all cylinders of the system.

Notice:

Master Key:

MK can operate lock no.(1-4)

Individual Key:

IK-1 can operate lock cylinders 1

IK-2 can operate lock cylinders 2

IK-3 can operate lock cylinders 3

IK-4 can operate lock cylinders 4

Grand master Key System

Is the concentration of different master-key systems as one system.  The grand-master-key passes all locks, the master-key passes all locks of the region, the single key passes only one cylinder.

Master Key:

MK-1 can operate lock no.(1.1 - 3.1)

MK-2 can operate lock no.(1.2 - 4.2)

MK-3 can operate lock no.(1.3 - 3.3)

Individual Key :

IK-1.1 can operate lock cylinder 1.1

IK-2.1 can operate lock cylinder 2.1

IK-3.1 can operate lock cylinder 3.1

EURO PROFILE CYLINDER

Knob Series

Knob No. 1	Knob No. 2	Knob No. 3

Double Entry Function	Single Entry Function

Privacy Function	Security & Safety Lock

Half Cylinder	Half Cylinder With Knob

| A (mm) | B (mm) | C (mm) |
| --- | --- | --- |
| 65 | 32.5 | 32.5 |
| 70 | 35 | 35 |
| 80 | 40 | 40 |
| 90 | 45 | 45 |
| 100 | 50 | 50 |
| 110 | 55 | 55 |
| 120 | 60 | 60 |

EURO PROFILE CYLINDER

| A (mm) | B (mm) | C (mm) |
| --- | --- | --- |
| 65 | 32.5 | 32.5 |
| 70 | 35 | 35 |
| 80 | 40 | 40 |
| 90 | 45 | 45 |
| 100 | 50 | 50 |
| 110 | 55 | 55 |
| 120 | 60 | 60 |

| A (mm) | B (mm) | C (mm) |
| --- | --- | --- |
| 65 | 32.5 | 32.5 |
| 70 | 35 | 35 |

| A (mm) | B (mm) | C (mm) |
| --- | --- | --- |
| 40 | 30 | 10 |
| 45 | 35 | 10 |

| A (mm) | B (mm) | C (mm) |
| --- | --- | --- |
| 65 | 32.5 | 32.5 |
| 70 | 35 | 35 |

Available Finishes:

AB – Antique Brass

PB – Polished Brass

SN – Satin Nickel